本试剂盒只能用于科学研究，不得用于医学诊断

小鼠CD24分子（CD24）ELISA科研试剂盒

使用说明书

试剂盒组成

| 名称 | 96孔配置 | 48孔配置 | 备注 |
| --- | --- | --- | --- |
| 微孔酶标板 | 8孔×12条 | 8孔×6条 | 无 |
| 标准品 | 0.3mL*6管 | 0.3mL*6管 | 无 |
| 检测抗体 | 6mL | 3mL | 无 |
| HRP酶结合物工作液 | 6mL | 3mL | 无 |
| 20×洗涤缓冲液 | 25mL | 15mL | 按说明书进行稀释 |
| 底物A | 6mL | 3mL | 无 |
| 底物B | 6mL | 3mL | 无 |
| 终止液 | 6mL | 3mL | 无 |
| 封板膜 | 2张 | 2张 | 无 |
| 说明书 | 1份 | 1份 | 无 |

注：标准品（S0-S5）浓度依次为：0、0.75、1.5、3、6、12 ng/mL。

检测原理

试剂盒采用双抗体两步夹心法酶联免疫吸附试验（ELISA）。往预先包被小鼠CD24分子（CD24）抗体的包被微孔中，依次加入标本、标准品、检测抗体，经过温育并洗涤。再加入HRP酶结合物工作液，经过温育并彻底洗涤。用底物TMB显色，TMB在过氧化物酶的催化下转化成蓝色，并在酸的作用下转化成最终的黄色。颜色的深浅和样品中的小鼠CD24分子（CD24）呈正相关。用酶标仪在450nm波长下测定吸光度（OD值），计算样品浓度。

样品收集、处理及保存方法

细胞培养上清：用无菌管收集，2-8℃条件离心20分钟（3000转/分），小心仔细收集上清，或存储在-20℃或-80℃备用。但应避免反复冻融。

细胞样本：

动物细胞：对于贴壁细胞，用适量预冷的PBS轻轻清洗细胞，并用胰蛋白酶消化分离细胞。以1000×g离心5分钟收集细胞（悬浮细胞可直接离心收集）。弃上清，用冷PBS清洗细胞3次。在浓度为1×107个细胞/mL的冷PBS中重新悬浮细胞。用超声破碎仪充分破碎细胞，以使细胞破坏并放出细胞内成份。2-8℃条件离心20分钟（3000转/分），随后小心仔细取上清液进行检测。

样本的要求

标本采集后尽早进行提取，提取后应尽快进行实验。若不能马上进行试验，样品在2-8℃可保存时应在6天内使用，否则必须在-20℃(≤1个月)或-80℃(≤2个月)保存，应避免反复冻融。保存过程中如有沉淀形成，上样检测前应再次离心。不能检测含NaN3的样品，因NaN3抑制辣根过氧化物酶的（HRP）活性。

自备物品

- 酶标仪（450nm）

- 高精度加样器及枪头：0.5-10μL、2-20μL、20-200μL、200-1000μL

- 37℃恒温箱

操作注意事项

- 试剂盒保存在2-8℃，使用前室温平衡20分钟。从冰箱取出的浓缩洗涤液会有结晶，这属于正常现象，水浴加热使结晶完全溶解后再使用。

- 实验中不用的板条应立即放回自封袋中，密封（低温干燥）保存。

- 浓度为0的S0号标准品即可视为阴性对照或者空白。

- 严格按照说明书中标明的时间、加液量及顺序进行温育操作。

- 所有液体组分使用前充分摇匀。

- 暂时用不到的酶标板条取下来放进铝箔袋备用，2-8℃保存。建议尽快使用。

试剂的准备

20×洗涤缓冲液的稀释：蒸馏水按1：20稀释，即1份的20×洗涤缓冲液加19份的蒸馏水。

洗板方法

- 手工洗板：甩尽孔内液体，每孔加满洗涤液，静置1min后甩尽孔内液体，在吸水纸上拍干，如此洗板5次。

- 自动洗板机：每孔注入洗液350μL，浸泡1min，洗板5次。

操作步骤

- 从室温平衡20min后的铝箔袋中取出所需板条，剩余板条用自封袋密封放回4℃。

- 设置标准品孔和样本孔，标准品孔各加不同浓度的标准品50μL，样本孔直接加入样本50μL。

- 标准品孔和样本孔中每孔加入检测抗体50μL，用封板膜封住反应孔，37℃水浴锅或恒温箱温育30min。

- 弃去液体，吸水纸上拍干，每孔加满洗涤液，静置1min，甩去洗涤液，吸水纸上拍干，如此重复洗板5次（也可用洗板机洗板）。

- 标准品孔和样本孔中每孔加HRP酶结合物工作液50μL，用封板膜封住反应孔，37℃水浴锅或恒温箱温育30min。

- 弃去液体，吸水纸上拍干，每孔加满洗涤液，静置1min，甩去洗涤液，吸水纸上拍干，如此重复洗板5次（也可用洗板机洗板）。

- 每孔加入底物A、B各50μL，37℃避光孵育15min。

- 每孔加入终止液50μL，15min以内，在450nm波长处测定各孔的OD值。

结果判断

- 绘制标准曲线：在Excel工作表中，以标准品浓度作横坐标，对应OD值作纵坐标，绘制出标准品线性回归曲线，按曲线方程计算各样本浓度值。拟合方程选择可以有多种形式，比如直线，对数，指数，二次函数，双对数，四参数等。

- 注意：此标曲图仅供参考，实际检测请以最终制作的标曲为准。

试剂盒性能

- 准确性：标准品线性回归与预期浓度相关系数R值，大于等于0.9900。

- 灵敏度：最低检测浓度小于0.75 ng/mL。

- 检测范围：0.75-12ng/mL。

- 特异性：不与其它可溶性结构类似物交叉反应。

- 重复性：板内、板间变异系数均小于15%。

- 贮藏：2-8℃，避光防潮保存。

- 有效期：6个月。

免责声明

- 试剂盒仅供研究使用，不得用于临床实验或人体实验，否则所产生的一切后果，由实验者承担，本公司概不负责。

- 严格按照说明书操作，实验者违反说明书操作，后果由实验者承担。

FOR RESEARCH USE ONLY.

NOT FOR USE IN DIAGNOSTIC PROCEDURES.

Mouse CD24 ELISA Kit instruction

Intended use

This CD24 ELISA kit is intended Laboratory for Research use only and is not for use in diagnostic or therapeutic procedures.The Stop Solution changes the color from blue to yellow and the intensity of the color is measured at 450 nm using a spectrophotometer. In order to measure the concentration of CD24 in the sample, this CD24 ELISA Kit includes a set of calibration standards. The calibration standards are assayed at the same time as the samples and allow the operator to produce a standard curve of Optical Density versus CD24 concentration. The concentration of CD24 in the samples is then determined by comparing the O.D. of the samples to the standard curve.

Sample collection and storages

Cell culture supernates and other biological fluids - Remove particulates by centrifugation and assay immediately or aliquot and store samples at -20℃or -80℃. Avoid repeated freeze-thaw cycles.

Note:  The samples shoule be centrifugated dequately and no hemolysis or granule was allowed.

Materials required but not supplied

1.  Standard microplate reader(450nm).

2.  Precision pipettes and Disposable pipette tips.

3.  37 ℃ incubator.

Precautions

1.  Do not substitute reagents from one kit to another. Standard, conjugate and microplates are matched for optimal performance. Use only the reagents supplied by manufacturer.

2.  Do not remove microplate from the storage bag until needed. Unused strips should be stored at 2-8°C in their pouch with the desiccant provided.

3.  Mix all reagents before using.

4.  Remove all kit reagents from refrigerator and allow them to reach room temperature ( 20-25°C).

Materials supplied

| Name | 96 determinations | 48 determinations |
| --- | --- | --- |
| Microelisa stripplate | 8*12strips | 8*6strips |
| Standard | 0.3mL*6tubes | 0.3mL*6tubes |
| Detection Antibody | 6.0mL | 3.0mL |
| HRP Streptavidin | 6.0mL | 3.0mL |
| 20X Wash solution | 25mL | 15mL |
| Chromogen Solution A | 6.0mL | 3.0mL |
| Chromogen Solution B | 6.0mL | 3.0mL |
| Stop Solution | 6.0mL | 3.0mL |
| Closure plate membrane | 2 | 2 |
| User manual | 1 | 1 |

Note: Standard (S0 → S5) concentration was followed by:0,0.75,1.5,3,6,12 ng/mL.

Reagent preparation

20×wash solution:Dilute with Distilled or deionized water 1:20.

Assay procedure

1.  Prepare all reagents before starting assay procedure. It is recommended that all Standards and Samples be added in duplicate to the Microelisa Stripplate.

2.  Add standard: Set Standard wells, testing sample wells. Add standard 50μL to standard well.

3.  Add Sample: Add testing sample 50μL to testing sample well.Add 50μL of Detection Antibody to each well.cover with an adhesive strip and incubate for 30 minutes at 37°C.

4.  Aspirate each well and wash, repeating the process four times for a total of five washes. Wash by filling each well with Wash Solution (300μL) using a squirt bottle, manifold dispenser or autowasher. Complete removal of liquid at each step is essential to good performance. After the last wash, remove any remaining Wash Solution by aspirating or decanting. Invert the plate and blot it against clean paper towels.

5.  Add 50μL of HRP Streptavidin to each well, cover with an adhesive strip and incubate for 30 minutes at 37°C.

6.  Aspirate each well and wash, repeating the process four times for a total of five washes. Wash by filling each well with Wash Solution (300μL) using a squirt bottle, manifold dispenser or autowasher. Complete removal of liquid at each step is essential to good performance. After the last wash, remove any remaining Wash Solution by aspirating or decanting. Invert the plate and blot it against clean paper towels.

7.  Add chromogen solution A 50μL and chromogen solution B 50μL to each well. Gently mix and incubate for 15 minutes at 37°C. Protect from light.

8.  Add 50μL Stop Solution to each well. The color in the wells should change from blue to yellow. If the color in the wells is green or the color change does not appear uniform, gently tap the plate to ensure thorough mixing.

9.  Read the Optical Density (O.D.) at 450 nm using a microtiter plate reader within 15 minutes.

Calculation of results

- This standard curve is used to determine the amount in an unknown sample. The standard curve is generated by plotting the average O.D. (450 nm) obtained for each of the six standard concentrations on the vertical (Y) axis versus the corresponding concentration on the horizontal (X) axis.

- First, calculate the mean O.D. value for each standard and sample. All O.D. values, are subtracted by the mean value of the zero standard before result interpretation. Construct the standard curve using graph paper or statistical software.

- To determine the amount in each sample, first locate the O.D. value on the Y-axis and extend a horizontal line to the standard curve. At the point of intersection, draw a vertical line to the X-axis and read the corresponding concentration.

- Any variation in operator, pipetting and washing technique, incubation time or temperature, and kit age can cause variation in result. Each user should obtain their own standard curve.

- The sensitivity by this assay is 0.75 ng/mL.

- Standard curve.

- Note: This standard curve graph is for reference only. For actual testing, please refer to the final prepared standard curve.

Storage：  2-8℃.

Validity： six months.

FOR RESEARCH USE ONLY; NOT FOR THERAPEUTIC OR DIAGNOSTIC APPLICATIONS! PLEASE READ THROUGH ENTIRE PROCEDURE BEFORE BEGINNING!